### TABLE 10

### LC-MS/MS conditions for the three assays on a single oral fluid sample.

|  |  |  | P63 |  | THC |  |  | D- and L- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scan type |  |  | MRM |  | MRM |  |  | MRM |
| Ion source |  |  | Turbo spray |  | Turbo spray |  |  | Turbo spray |
| Probe position |  |  | X = 5.15, Y = 5.4 |  | X = 5.15, Y = 6.40 |  |  | X=5.00, Y=5.2 |
| Polarity: |  |  | Positive |  | Negative |  |  | Positive |
| Run duration |  |  | 8 min |  | 1.5 min |  |  | 11 min |
| Settling time (msec) |  |  | 5 |  | 0 |  |  | 0 |
| Pause time (msec) |  |  | 5 |  | 5.0007 msec |  |  | 7.007 msec |
| Curtain gas |  |  | 35 |  | 20 |  |  | 35 |
| CAD gas |  |  | 8 |  | 4 |  |  | 4 |
| ISV (V) |  |  | 5000 |  | -4000 |  |  | 5000 |
| Temperature (°C) |  |  | 500 |  | 400 |  |  | 500 |
| Ion Source Gas 1 (GS1) |  |  | 50 |  | 50 |  |  | 50 |
| Ion Source Gas 2 (GS2) |  |  | 50 |  | 60 |  |  | 50 |
| Q1/Q3 resolution |  |  | unit/unit |  | unit/unit |  |  | unit/unit |
| CEM (V) |  |  | 2600 |  | 2400 |  |  | 2600 |

### Note: multiple reaction monitoring (MRM); collision gas (CAD); ion source voltage (ISV); channel electron multiplier (CEM)